Safety:

- Ensure that the UTV is safely parked on level ground with the ignition off.

- Wear all appropriate PPE for the following procedures.

Note:

- Leave all hardware loose until the entire assembly is in its final position.

Assembly Components:

Ensure that you have all of these components before beginning the installation.

1 x Square Tube Bumper

2 x Mounting Brackets (left and right)

1 x Accent Plate

2 x Light Hole Cover Plates

1 x Hardware Kit

Installation Procedure:

- Locate the mounting holes on the cargo bed frame on the underside of the cargo bed.

Figure 1: Right Side Bed Frame Holes

- Drill out the mounting holes to accommodate the 5/16” bolts. The small wiring harness on the right side must be relocated and secured with a zip tie or other secure method.

Figure 2: Drilled Out Bed Frame Holes

- Mount the left and right brackets to the drilled-out bed frame holes using the included 5/16” bolts and corresponding nuts. Reference Figure 4 for the correct placement.

Figure 3: Right Side Mounting Bracket

Figure 4: Mounting Brackets

- Mount the accent plate to the inside of the bumper using the included machine screws and their corresponding nuts.

Figure 5: Accent Plate Inside the Bumper

Figure 6: Accent Plate Outside Bumper

- Mount the bumper to the brackets with the included carriage bolts and nuts.

Figure 7: Bumper Mounted on Brackets

- At this point you may either install the light hole covers over the openings in the bumper, or install lights into the bumper (you may purchase these lights at www.baddawgaccessories.com). Figure 8 illustrates the light hole cover installation, and Figure 9 shows lights properly installed.

Figure 8: Light Hole Covers

Figure 9: Bumper with Lights

- Ensure that all hardware is tightened securely before operating your machine.

Figure 10: Installation Complete