Safety:

- Ensure that the UTV is safely parked on level ground with the ignition off.

- Wear all appropriate PPE for the following procedures.

Note:

- Leave all hardware loose until the entire assembly is in its final position.

Assembly Components:

Ensure that you have all of these components before beginning the installation.

2 x CV Joint Protectors

2 x Mounting Brackets

1 x Hardware Kit

Figure 1: Kit Contents

Installation:

Note: The following procedures show the installation performed on the front-passenger-side of the machine.

- Remove the stock CV joint protector. Remove the two nuts and bolts holding the joint protector to the lower a-arm.

Figure 2: CV Joint Protector

Figure 3: Stock CV Joint Bolts

- Match the correct metal CV joint protector mounting holes to the correct a-arm mounting location.

Figure 4: Joint Protector Mounting Location

Figure 5: Joint Protector Mounting Holes

Figure 6: Passenger Side Indicate

- Position the new protector on the lower a-arm and insert the included hardware. Make sure that the carriage bolt is inserted into the square hole.

Figure 7: Mounting Bolts and Corresponding Nuts

Figure 8: Protector Mounted (Carriage Bolt Indicated)

- Match the mounting bracket to the protector on the top side of the lower a-arm. Secure all of the bolts using the matching nuts. Make sure that the carriage bolt is secured with a washer and nylock nut.

Figure 9: Top Bracket (Carriage Bolt Indicated)

- Repeat these procedures on the opposite side, ensuring that the hardware mirrors the passenger side setup.